Functional Specification MDF Converter Functions

February 23 2026 IH Development Authored by: Ingemar Hedvall

Innehåll

Summary	3

Converter Functionality	4

Converter Task	4

Bus Master Task	4

MDF Converter Object	4

MDF Version	5

Storage Options	5

Overwrite Option	5

Implementation	5

Validating Task	7

Implementation	7

Sorting Task	8

Implementation	8

CAN Sorting Task	9

Implementation	9

Cutting Task	10

Implementation	10

Converting to Other File Formats	11

Implementation	11

Summary

This document describes the converter functions for the C++ MDF library. The following converter tasks are described.

- Validate. Validates an MDF file.

- Sorting. Create a new so-called sorted MDF file.

- CAN Sorting (Signal). Converts CAN messages to DBC signals.

- Cutting. Remove samples from a file based upon time range.

- Convert to CSV File(s). Create one or more CSV files.

- Convert to HDF5 File(s). Create one or more HDF5 files.

- Convert to Parquet File(s). Create one or more Parquet files.

Converter Functionality

MDF Task

A MDF task is just a simple wrapper around MDF reader and MDF writer objects. It is like a Bus Master Task, but it doesn’t have the abstract source and destination objects. This makes the MDF task easy to implement but it shouldn’t be used for complex designs.

Bus Master Task

The Bus Master task defines 4 abstract objects.

- Environment. Defines a simple message bus or a TX/RX bus.

- Database. Optional object that holds a database file for example a DBC file.

- Source. Defines the input data source for example an MDF file.

- Destination. Defines the output data destination.

The environmental object is a singleton that defines either a simple one directional message bus or a TX/RX message bus. Several databases, sources and destinations can be attached to the bus. There exist a simple GUI for debugging and simulating different combinations.

If a task involves bus messages typically using the CAN protocol, the converter task should be based upon the bus master library. A bus message task can have several databases, sources and destinations.

MDF Converter Object

The MDF converter object is just a specialization of the MDF writer object. It has all the MDF writer functionality, but the internal sample queue is not saved periodically. Instead, it checks the temporary queue size and saves the samples in 4MB blocks. Currently, there is a separate writing thread for saving samples to the disc, but a better design should have been to run both adding and saving samples in the same thread.

The MDF writer opens and closes the file, periodically for each sample data save. This makes the file finalized and other external applications can read the file. A converter doesn’t need this functionality. Instead, it is better to just keep the file open and close it at the end.

MDF Version

Saving a specific MDF version is a wanted functionality. This functionality is supported but a more user-friendly interface is requested.

Storage Options

Variable length data (VLSD) can be stored in 3 different ways. This functionality is already available by the MDF converter object.

Overwrite Option

There is an issue when creating new file if that file already exists or if it replaces the existing file. This is a common error for the end-user. Maybe add an overwrite option that must be set otherwise the task fails.

Note that a converter task always creates a temporary file and copy that file to its destination path. The purpose of this is both speed issue and a problem with NAS file system write through option. An MDF file should not be created directly on an NAS server. This can fail depending on how the NAS does its write optimization. This generates randomized errors.

Implementation

Note that some converter tasks are not implemented in the current MDF C++ library due to dependency of external libraries. Dependencies of DBC, HDF5, CSV and Parquet are typical examples.

A converter task can be wrapped into a command line executable like the MDF-to-CSV application, but the actual task implementation is created in a library giving the end-user the choice to include the code or to run a separate executable.

Validating Task

This task should validate MDF file and mark the file as valid or not. The validation logic is dependent on the test object why this task is intended to be modified by the end-user. It will not change any configuration or values in the existing file.

This base task will have the following sub-tasks.

- Scan through all samples and check that they are all valid.

- Append a file history block with the validation result.

The task is implemented in the current C++ library as it doesn’t have any dependency on third party libraries.

Implementation

The task shall be implemented in the C++ Library. It is optional if the task should be wrapped into a command line executable.

The validation, sorting and cutting tasks can be wrapped into an MDF-to-MDF command line executable like the MDF-to-CSV task.

Sorting Task

The MDF file is considered sorted if each DG block only has one CG block. This enables faster reading of the file as all samples are in known places in the file. Some readers can use this feature when fetching individual sample values. Most readers assume that the file is unsorted which in turn generates longer read times.

The 4.2 version of the ASAM MDF introduced special LD/DV that in theory should give the MDF the same read speed as Parquet files. Not found any reader or writer software that can handle this correctly. It better to wait with this LD/DV functionality until there is a way of validating it.

Note that some CG-VLSD storage is not supported as it requires extra CG blocks.

Implementation

The task functionality can be implemented in the existing MDF C++ library and optionally wrapped into an MDF-to-MDF command line executable.

CAN Sorting Task

If the file is according to the ASAM MDF bus standard, the file can be sorted on the message ID. One DG and CG block for each message ID. If a DBC file is supplied, each message can be split into signal values.

Multiplexed DBC signals are problematic as they generate less samples than messages. A way to solve this is to save the last reported signal value but this generates invalid values until the signal is updated.

When a DBC signal requires more than 8 bytes of data, the DBC needs to specify which transport protocol it uses. Most of the time, the DBC file is missing this, so optional the CAN transport protocol needs to be specified.

Other bus types like LIN protocol can be supported but each bus has its own type of configuration file which requires some serious development themself.

Note that the messages need to be read twice. The first read defines which message ID’s that exists in the file and number of samples/messages. This information is used to configure the sorted file. The second read saves the sample data as usual.

Implementation

The task will not be implemented in the MDF C++ library as it has a dependency on the DBC library. Instead, it will be using the Bus Master library.

Cutting Task

The main purpose is to remove samples from an MDF file and create a smaller file with a subset of the original samples. The range is specified by using the master channel which normally is relatively time range.

- First the original MDF file read its configuration. This configuration is reused by the new file. This is simply done by resetting all blocks file positions. The latter function(s) needs to be added.

- Next is to do an ordinary read of all master channels data. This defines which min and max samples to copy per CG group.

- Finally, all samples are read and range-copied to the new file.

The task needs an option that defines if the relative master time should be reset to start at zero. If enabled, the absolute start time needs to be modified.

Implementation

The task shall be implemented in the existing MDF C++ library. This task should be wrapped into the MDF-to-MDF executable.

Converting to Other File Formats

These tasks are developed in their own repository as they are dependent on other external libraries. A common feature is that most external file formats are missing meta-data information that are lost during the conversion.

Most external file formats can only handle one CG group. If the original MDF file consists of several CG groups, many output files are generated. The MDF-to-CSV has some automatic naming standards that can be reused.

The problem with MDF files is that it stores sample by sample. The disadvantage comes when retrieving values, almost the entire file needs to be read.

The Parquet file format uses column storage instead. This enables fast fetching of values. A typical Data Lake application has an index database for the meta-data while the sample data are stored in Parquet files.

The LD/DV storage makes it possible to read sample data much faster. The disadvantage is that VLSD data needs to be stored in SD block and compression might slow down the read. In theory this storage format can be used in Data Lake applications. Sorted MDF files and LD/DV MDF files might need specialized readers that don’t need to read the entire file.

Implementation

The current MDF C++ library already has too many options that generate some maintenance issues. It is recommended to create a separate repository for each format,